EX-1682  FIELD PROGRAMMING SOFTWARE

For the IC-M58

Copyright 1991-1994 Icom Inc.

Portions of this software are copyrighted by Microsoft Corporation 1982-1988

**                                                       **

**  This program is designed to perform cloning for the  **

**  IC-M58 VHF MARINE TRANSCEIVER only.                  **

**  Before using the program, make a backup copy of the  **

**  original disk.                                       **

**                                                       **

-------------------------------------------------------------------------------

TABLE OF CONTENTS

-------------------------------------------------------------------------------

1.  PROGRAM START UP AND SHUT DOWN ...................  2

2.  OPERATION ........................................  2

3.  SCREEN DESCRIPTIONS ..............................  3

COMMON SCREEN ..................................  3

MEMORY CHANNEL SCREEN ..........................  7

INT CHANNEL and USA CHANNEL SCREENS ............ 10

PRIVATE CHANNEL SCREENS ........................ 12

FREE CHANNEL SCREEN ............................ 14

WX CHANNEL SCREEN .............................. 16

EXPERT SCREEN .................................. 17

4.  CLONING OPERATION ................................ 20

5.  FILE SAVING AND LOADING .......................... 22

6.  PRINTING ......................................... 24

- 1 -

-------------------------------------------------------------------------------

1. PROGRAM START UP AND SHUT DOWN

-------------------------------------------------------------------------------

1) Boot up DOS.

2) Insert the EX-1682 backup disk.

3) Type the start command:

**                                       **

**  EX1682 /(version) /(option) [Enter]  **

**                                       **

1. USA, UK, EUR, HOL, or FRA can be used as "(version)".

2. When the version is specified, the corresponding

default settings are read.

3. When no version is specified, the previously

saved contents are called.

4. When "expert" is used as "(option)," the EXPERT screen is

available for programming additional items.

5. When a file name is used instead of "(version)," the specified file

is loaded at startup.

4) The COMMON screen appears.

**                                                              **

**  The loaded file name appears in the bottom left corner of   **

**  the screen.  "NONAME.ICF" appears when no data file is      **

**  loaded.                                                     **

**                                                              **

5) When programming is completed:

push [Esc] (indicates the TOP menu)

then [F] (selects file SUB menu)

then [S] (saves the set data; if not necessary skip this operation.)

then [Q] (quits the program)

-------------------------------------------------------------------------------

2. OPERATION

-------------------------------------------------------------------------------

The EX-1682 has a TOP menu and 9 screens (including the EXPERT screen)

for programming.  All screens or operations can be accessed from the TOP menu.

A SUB menu appears under the TOP menu when the item in the top menu is selected

with the [Arrow] and [Enter] keys.  The TOP menu appears when the [Esc] key is

pushed.

1) After the program starts up, push the [Esc] key to indicate the top menu.

2) Push a highlighted character; or the [Arrow] and [Enter] keys to select

the desired item.

- A SUB menu appears under the selected item.

3) Push a highlighted character; or the [Arrow] key and [Enter] keys to

select the desired screen.

- The desired screen appears.

4) Select the desired item with an [Arrow] key, then push [Space] or the

digit keys to set the item.

- See descriptions of each screen for details.

- 2 -

-------------------------------------------------------------------------------

3. SCREEN DESCRIPTIONS

-------------------------------------------------------------------------------

////// COMMON SCREEN //////

+[COMMON]---------+------------++-----------------+-----------+

| TITLE           |            || TITLE           |           |

+-----------------+------------++-----------------+-----------+

| DSC ID          |  OFF       || T.O.T. (Sec)    |       300 |

| DSC TX on DUPch |  INHIBIT   || BEEP            |  ON       |

| SENSOR MODE     |  DISTRESS  || POWER SAVE      |  OFF      |

| INDIVIDUAL ADDR |  OFF       || LCD BACKLIGHT   |  3        |

| ATIS ID         |  OFF       || WX ALERT        |  OFF      |

| ATIS INTERVAL   |       299  || SCRAMBLER CODE  |         0 |

| TX without ATIS |  ACCEPT    || U TIME DIFFER   |      0:00 |

+-----------------+------------++-----------------+-----------+

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A sub menu appears under the TOP menu.

3) Select "Common" with the [C] key, or the [Arrow] and [Enter] keys.

- The COMMON screen appears.

DSC ID

The authorized DSC code of the transceiver can be programmed in this

item.  This programmed DSC code is used for the distress call,

individual receive, etc.

**                                                            **

** IMPORTANT:  If the wrong code is programmed in this item,  **

**             distress calls will not be accepted and shore  **

**             stations may be confused.                      **

**                                                            **

Selectable value:     OFF or 0000000000 to 9999999999

(Only 10-digit numbers accepted.)

Selection method:     Push the [Space] key to turn the item OFF or;

Push digit keys, then [Enter] for code entry.

DSC TX on DUPch

Usable channels for the all ships call can be selected as any or simplex

channels only.  Even when "simplex only" is selected, the all ships call

can be used on some "A" channels (e.g. channel 88A).   The all ships call

transmits the channel data without "A" information, therefore, true

simplex channels (e.g. channel 10, 11, etc.) must be used for an all

ships call.

Selectable value:    ACCEPT or INHIBIT

Selection method:    Push the [Space] key to toggle the selection.

- 3 -

SENSOR MODE

This item determines whether a DSC distress transmission or individual

code transmission occurs when a sensor is triggered.

Selectable value:    DISTRESS or INDIVIDUAL

Selection method:    Push the [Space] key to toggle the selection.

INDIVIDUAL ADDR

This item sets the transmit code for individual calls.

Selectable value:    OFF or 0000000000 to 9999999999

Selection method:    Push the [Space] key to turn the item OFF or;

Push digit keys, then [Enter] for address entry.

(Only 10-digit numbers accepted.)

ATIS ID

The authorized ATIS code of the transceiver can be programmed in this

item.  This programmed ATIS code is used for ATIS encode and decode.

**                                                            **

** IMPORTANT:  If the wrong code is programmed in this item,  **

**             your ATIS code will confuse other ships.       **

**                                                            **

Selectable value:     OFF or 0000000000 to 9999999999

(Only 10-digit numbers accepted.)

Selection method:     Push the [Space] key to turn the item OFF or;

Push digit keys, then [Enter] for code entry.

ATIS INTERVAL

The ATIS code is transmitted not only at the end of a transmission but

also at intervals while transmitting for long periods.  The interval of

the ATIS transmission can be set.

Selectable value:    1 to 999 sec.

Selection method:    Push digit keys, then [Enter] for time entry.

TX without ATIS

A transmission can be inhibited when no ATIS code is programmed.

If this setting is set to "INHIBIT" without ATIS programming, the

transceiver can receive only.

Selectable value:    ACCEPT or INHIBIT

Selection method:    Push [Space] to toggle the selection.

- 4 -

T.O.T.(Sec)

The time-out-timer function inhibits long transmissions.  The timer

starts counting from the beginning of a transmission.

Selectable value:    OFF or 1 to 999 sec.

Selection method:    Push the [Space] key for OFF or;

Push digit keys, then [Enter] for time entry.

BEEP

Beep tones provide convenient audible confirmation when switches are

pushed.  The beep tones can be turned off when necessary.  This setting

can also be performed using the transceiver's SET mode.

Selectable value:    ON or OFF

Selection method:    Push [Space] to toggle the selection.

POWER SAVE

This item turns the transceiver's power saver function ON or OFF. This

setting can also be performed using the transceiver's SET mode.

Selectable value:    ON or OFF

Selection method:    Push the [Space] key to toggle the selection.

LCD BACKLIGHT

The display backlight intensity can be turned ON and OFF, and the intensity

can be selected from 3 levels.  This setting can also be performed from

the transceiver's front panel.

Selectable value:    OFF or 1 to 3   (3 is lightest)

Selection method:    Push [Space] or [Backspace] to select

1 to 3 or OFF, or

Push a digit key for backlight level entry.

WX ALERT

This item sets the transceiver's weather alert function ON or OFF. This

setting can also be performed using the transceiver's front panel.

Selectable value:    ON or OFF

Selection method:    Push [Space] to toggle the selection.

@@@@@@@@@@@@@@@@@@@- 5 -

SCRAMBLER CODE

A scrambler unit (UT-79) can be used with the transceiver.  When

communicating using the voice scrambler function, remember that all members

of a group must use the same scramble code. This setting can also be

performed using the transceiver's front panel.

Selectable value:    0 to 127

Selection method:    Push digit keys, then [Enter] for code entry.

U TIME DIFFER

Time difference from UTC (Universal Time Coordinated).  This item can also

be set using the transceiver's SET mode.

Selectable value:    -13:00 to 13:00

Selection method:    Push digit keys (incl. "-" and ":"),

then [Enter].

- 6 -

////// MEMORY CHANNEL SCREEN //////

+[MEMORY CHANNEL]-----+-----+-----+-----+------+

|CH  | CHANNEL  |LOCK-|Tx   |LOW  | DUP | SCRAM|

| No.|          | OUT | INH | PWR |     | BLER |

+----+----------+-----+-----+-----+-----+------+

|  0 | USA  16  |     |     |     |     |      |

|  1 | INHIBIT  |     |     |     |     |      |

|  2 | INHIBIT  |     |     |     |     |      |

~                                              ~

~                                              ~

| 23 | INHIBIT  |  -  |     |     |     |      |

|CALL| USA  09  |  -  |     |     |     |      | These bottom 3 lines of

|CH16|      16  |  -  |     |     |  -  |   -  | the memory screen only

|DSCM| INT  70  |  -  |  -  |  -  |  -  |   -  | appear when "expert" is

|DSCS| INT  16  |  -  |  -  |  -  |  -  |   -  | included in the startup

+----+----------+-----+-----+-----+-----+------+ command (p. 2).

This screen allows you to set all memory channel and call channel contents.

DSCM and DSCS stand for the DSC main and DSC sub channels respectively.  The

DSC main channel is used for a distress call transmission and the DSC sub

channel is used for channel selection after the distress call transmission.

**                                                            **

**  IMPORTANT:  NEVER CHANGE THE DSC MAIN CHANNEL AND [CH16]  **

**              CALLING CHANNEL EVEN THOUGH THESE CHANNELS    **

**              CAN BE SELECTED ON THIS SCREEN.               **

**                                                            **

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A sub menu appears under the TOP menu.

3) Select "Memory" with the [M] key, or the [Arrow] and [Enter] keys.

- The MEMORY CHANNEL screen appears.

CHANNEL

The programmed channels appear when the MEMORY mode is selected.

1) Select the desired channel in the CHANNEL column with an [Arrow] key.

2) Push the [Space] or [Backspace] key to select the desired channel group:

**  INT  16  **

**  USA  16  **

**  WX    1  **

**  INHIBIT  **

3) Push digit keys to enter the desired marine channel number,

then push the [Enter] key.

- "A" can be input as a channel number.

- For a channel number only (i.e. without "INT" or "USA"), enter the

channel number when "INHIBIT" is displayed.

4) Push [Enter] to enter the channel data.

- The channel is highlighted and the [Arrow] and [Esc] keys can

be used.

- 7 -

LOCKOUT

The selected channels are locked out during memory scan. This setting can also

be performed via the transceiver's front panel.

1) Select the desired channel in the LOCKOUT column with an [Arrow] key.

2) Push the [Space] key to select lockout or non-lockout.

- "L" appears when lockout is specified and no indicator appears when

lockout is canceled.

Tx INH

The selected channels can only receive; transmit is inhibited.

1) Select the desired channel in the Tx INH column with an [Arrow] key.

2) Push the [Space] key to select transmit inhibit or transmit non-inhibit.

-"I" appears when transmit inhibit is specified and no indicator appears

when transmit inhibit is canceled.

LOW POWER

The high/low power selection is disabled and only low power can be used on

the selected channel.

1) Select the desired channel in the LOW PWR column with an [Arrow] key.

2) Push the [Space] key to select low or high power.

- "P" appears when low power is specified and no indicator appears when

high power is specified.

**                                                                      **

**  NOTE:  Low output power must be set when programming USA channels   **

**         13 and 67 (momentary high power channels).  The output power **

**         cannot be selected from the transceiver's front panel. High  **

**         power can be selected during transmission ONLY when low      **

**         power is set on these channels.                              **

**                                                                      **

DUP

Duplex information must be set for each channel (except when programming "A"

channels).  Otherwise, an incorrect frequency is transmitted.

1) Select the desired channel in the DUP column with an [Arrow] key.

2) Push the [Space] key to select duplex or simplex.

- "D" appears when duplex is specified and no indicator appears when

simplex is specified.

- "-" appears when "A" channels (e.g. channel 88A) are selected.

In this case, a duplex setting cannot be specified.

- 8 -

SCRAMBLER

The scrambler function is turned ON for the selected channels. This setting

can also be performed using the transceiver's front panel.

1) Select the desired channel in the SCRAMBLER column with an [Arrow] key.

2) Push the [Space] key to select scrambler function ON or OFF.

-"S" appears when the scrambler function is turned ON.

-No indication appears when the scrambler function is turned OFF.

- 9 -

//////  INT CHANNEL and USA CHANNEL SCREENS  //////

+[INT CHANNEL]---+-----+------+-----++----+-----+-----+-----+------+-----+

|CH  |CH   |LOCK-|Tx   | LOW  | DUP ||CH  |CH   |LOCK-|Tx   | LOW  | DUP |

| No.| INH |  OUT| INH |  PWR |     || No.| INH |  OUT| INH |  PWR |     |

+----+-----+-----+-----+------+-----++----+-----+-----+-----+------+-----+

|  1 |     |     |     |      |  D  || 60 |     |     |     |      |  D  |

|  1A|     |     |     |      |  -  || 60A|     |     |     |      |  -  |

|  2 |     |     |     |      |  D  || 61 |     |     |     |      |  D  |

~                                                                        ~

~                                                                        ~

| 28A|  I  |     |     |      |  -  || 87A|     |     |     |      |  -  |

|    |  -  |  -  |  -  |  -   |  -  || 88 |     |     |     |      |  D  |

|    |  -  |  -  |  -  |  -   |  -  || 88A|     |     |     |      |  -  |

+----+-----+-----+-----+------+-----++----+-----+-----+-----+------+-----+

+[USA CHANNEL]---+-----+------+-----++----+-----+-----+-----+------+-----+

|CH  |CH   |LOCK-|Tx   | LOW  | DUP ||CH  |CH   |LOCK-|Tx   | LOW  | DUP |

| No.| INH |  OUT| INH |  PWR |     || No.| INH |  OUT| INH |  PWR |     |

+----+-----+-----+-----+------+-----++----+-----+-----+-----+------+-----+

|  1 |  I  |     |     |      |     || 60 |  I  |     |     |      |     |

|  1A|     |     |     |      |  -  || 60A|     |     |     |      |  -  |

|  2 |     |     |     |      |     || 61 |  I  |     |     |      |     |

~                                                                        ~

~                                                                        ~

| 28A|  I  |     |     |      |  -  || 87A|  I  |     |     |      |  -  |

|    |  -  |  -  |  -  |  -   |  -  || 88 |     |     |     |      |  D  |

|    |  -  |  -  |  -  |  -   |  -  || 88A|     |     |     |      |  -  |

+----+-----+-----+-----+------+-----++----+-----+-----+-----+------+-----+

When the international or USA channel group is selected with the transceiver,

the channels in the screen table can be used.  When "I" (Inhibit) is selected

in the "CH INH" column, the selected channels do not appear in the selected

channel group.

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A sub menu appears under the TOP menu.

3) Select "INT  ch" or "USA  ch" with the [I] or [U] key, or the [Arrow]

and [Enter] keys.

- The INT CHANNEL or USA CHANNEL screen appears.

CH INH

The selected channels do not appear during channel selection.

1) Select the desired channel in the CH INH column with an [Arrow] key.

2) Push the [Space] key to toggle the "I" or no indication selection.

- "I" means inhibit.

- No indication means the channel appears and can be used during

operation.

- 10 -

LOCKOUT

The selected channel is locked out during scan. This setting can also be

performed using the transceiver's front panel.

1) Select the desired channel in the LOCKOUT column with an [Arrow] key.

2) Push the [Space] key to select lockout or non-lockout.

- "L" appears when lockout is specified and no indicator appears when

lockout is canceled.

Tx INH

The selected channel can receive only; transmit is inhibited.

1) Select the desired channel in the Tx INH column with an [Arrow] key.

2) Push the [Space] key to select transmit inhibit or transmit non-inhibit.

- "I" appears when transmit inhibit is specified and no indicator appears

when transmit inhibit is canceled.

LOW POWER

The high/low power selection is disabled and only low power can be used on

the selected channel.

1) Select the desired channel in the LOW PWR column with an [Arrow] key.

2) Push the [Space] key to select low or high power.

- "P" appears when low power is specified and no indicator appears when

high power is specified.

**                                                                      **

**  NOTE:  Low output power must be set on USA channels 13 and 67       **

**         (momentary high power channels).  The output power cannot    **

**         be selected from the transceiver's front panel.  High power  **

**         can be selected during transmission ONLY when low power is   **

**         set on these channels.                                       **

**                                                                      **

DUP

Duplex information must be set for each channel (except when programming "A"

channels).  Otherwise, an incorrect frequency is transmitted.

1) Select the desired channel in the DUP column with an [Arrow] key.

2) Push the [Space] key to select duplex or simplex.

- "D" appears when duplex is specified and no indicator appears when

simplex is specified.

- "-" appears when "A" channels (e.g. channel 88A) are selected.

In this case, a duplex setting cannot be specified.

- 11 -

////// PRIVATE CHANNEL SCREENS //////

+[INT PRIVATE CHANNEL]-+-----+-----++----+-----+-----+-----+-----+-----+

|CH  |CH   |LOCK-|Tx   |LOW  | DUP ||CH  |CH   |LOCK-|Tx   |LOW  | DUP |

| No.| INH |  OUT| INH | PWR |     || No.| INH |  OUT| INH | PWR |     |

+----+-----+-----+-----+-----+-----++----+-----+-----+-----+-----+-----+

| 29 |  I  |     |     |     |  D  || 89 |  I  |     |     |     |  D  |

| 29A|  I  |     |     |     |  -  || 89A|  I  |     |     |     |  -  |

| 30 |  I  |     |     |     |  D  || 89 |  I  |     |     |     |  D  |

~                                                                      ~

~                                                                      ~

| 58A|  I  |     |     |     |  -  ||  0A|  I  |     |     |     |  -  |

| 59 |  I  |     |     |     |  D  ||    |  -  |  -  |  -  |  -  |  -  |

| 59A|  I  |     |     |     |  -  ||    |  -  |  -  |  -  |  -  |  -  |

+----+-----+-----+-----+-----+-----++----+-----+-----+-----+-----+-----+

+[USA PRIVATE CHANNEL]-+-----+-----++----+-----+-----+-----+-----+-----+

|CH  |CH   |LOCK-|Tx   |LOW  | DUP ||CH  |CH   |LOCK-|Tx   |LOW  | DUP |

| No.| INH |  OUT| INH | PWR |     || No.| INH |  OUT| INH | PWR |     |

+----+-----+-----+-----+-----+-----++----+-----+-----+-----+-----+-----+

| 29 |  I  |     |     |     |  D  || 89 |  I  |     |     |     |  D  |

| 29A|  I  |     |     |     |  -  || 89A|  I  |     |     |     |  -  |

| 30 |  I  |     |     |     |  D  || 89 |  I  |     |     |     |  D  |

~                                                                      ~

~                                                                      ~

| 58A|  I  |     |     |     |  -  ||  0A|  I  |     |     |     |  -  |

| 59 |  I  |     |     |     |  D  ||    |  -  |  -  |  -  |  -  |  -  |

| 59A|  I  |     |     |     |  -  ||    |  -  |  -  |  -  |  -  |  -  |

+----+-----+-----+-----+-----+-----++----+-----+-----+-----+-----+-----+

The transceiver has preset private channels.  When "I" (Inhibit) is cleared

in the "CH INH" column, the selected channels appear in the international or

USA channel group.

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or an [Arrow] and [Enter] keys.

- A SUB menu appears under the TOP menu.

3) Select the desired channel screen:

- For INT (PRV)ch screen, push [N], or the [Arrow] and [Enter] keys.

- For USA (PRV)ch screen, push [S], or the [Arrow] and [Enter] keys.

CH INH

The selected channels ("I" is indicated) do not appear during channel

selection.

1) Select the desired channel in the CH INH column with an [Arrow] key.

2) Push the [Space] key to toggle the "I" or no indication selection.

- "I" means Inhibit.

- No indication means the channel appears and can be used

during operation.

- 12 -

LOCKOUT

The selected channel is locked out during scan. This setting can also be

performed using the transceiver's front panel.

1) Select the desired channel in the LOCKOUT column with an [Arrow] key.

2) Push the [Space] key to select lockout or non-lockout.

- "L" appears when lockout is specified and no indicator appears when

lockout is canceled.

Tx INH

The selected channel can receive only; transmit is inhibited.

1) Select the desired channel in the Tx INH column with an [Arrow] key.

2) Push the [Space] key to select transmit inhibit or transmit non-inhibit.

- "I" appears when transmit inhibit is specified and no indicator appears

when transmit inhibit is canceled.

LOW POWER

The high/low power selection is disabled and only low power can be used on

the selected channel.

1) Select the desired channel in the LOW PWR column with an [Arrow] key.

2) Push the [Space] key to select low or high power.

- "P" appears when low power is specified and no indicator appears when

high power is specified.

DUP

Duplex information must be set for each channel (except when programming "A"

channels).  Otherwise, an incorrect frequency is transmitted.

1) Select the desired channel in the DUP column with an [Arrow] key.

2) Push the [Space] key to select duplex or simplex.

- "D" appears when duplex is specified and no indicator appears when

simplex is specified.

- "-" appears when "A" channels (e.g. channel 88A) are selected.

In this case, a duplex setting cannot be specified.

- 13 -

////// FREE CHANNEL SCREEN //////

+[FREE CHANNEL]--+-----+-----+-----------+-----------+

|CH  |CH   |LOCK-|Tx   |LOW  | RX FREQ.  |  TX FREQ. |

| No.| INH | OUT | INH | PWR |    [MHz]  |     [MHz] |

+----+-----+-----+-----+-----+-----------+-----------+

| F1 |  I  |     |     |     | --------- | --------  |

| F2 |  I  |     |     |     | --------- | --------  |

| F3 |  I  |     |     |     | --------- | --------  |

~                                                    ~

~                                                    ~

| P7 |  I  |     |     |     | 156.80000 | 156.80000 |

| P8 |  I  |     |     |     | 158.00000 | 158.00000 |

| P9 |     |     |     |     | 158.00000 | 158.00000 |

+----+-----+-----+-----+-----+-----------+-----------+

The transceiver has preset and programmable free channels. Channels F1 to F7,

L1 to L3, C1 to C4 and E1 are preset (receive and transmit frequencies cannot

be programmed). Channels P1 to P9 can be programmed with receive and transmit

frequencies.

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A SUB menu appears under the TOP menu.

3) Select "FREE ch" with the [F] key, or the [Arrow] and [Enter] keys.

CH INH

The selected channels ("I" is indicated) do not appear during channel

selection.

1) Select the desired channel in the CH INH column with an [Arrow] key.

2) Push the [Space] key to toggle the "I" or no indication selection.

- "I" means Inhibit.

- No indication means the channel appears and can be used

during operation.

LOCKOUT

The selected channel is locked out during scan. This setting can also be

performed using the transceiver's front panel.

1) Select the desired channel in the LOCKOUT column with an [Arrow] key.

2) Push the [Space] key to select lockout or non-lockout.

- "L" appears when lockout is specified and no indicator appears when

lockout is canceled.

- 14 -

Tx INH

The selected channel can receive only; transmit is inhibited.

1) Select the desired channel in the Tx INH column with and [Arrow] key.

2) Push the [Space] key to select transmit inhibit or transmit non-inhibit.

- "I" appears when transmit inhibit is specified and no indicator appears

when transmit inhibit is canceled.

LOW POWER

The high/low power selection is disabled and only low power can be used on

the selected channel.

1) Select the desired channel in the LOW PWR column with an [Arrow] key.

2) Push the [Space] key to select low or high power.

- "P" appears when low power is specified and no indicator appears when

high power is specified.

RX FREQ. [MHz]/TX FREQ. [MHz]

Receive and transmit frequencies can be set here (channels P1 to P9 only).

1) Select the desired channel in the RX FREQ. [MHz] or TX FREQ. [MHz] column

with an [Arrow] key.

2) Push the digit keys, then push [Enter] to input a receive or transmit

frequency.

- 100.00000 to 199.9950 MHz can be input.

**                                                              **

** NOTE: Even though out-of-range frequencies can be set, the   **

**       transceiver may not accept such frequencies.           **

**                                                              **

- 15 -

////// WX CHANNEL SCREEN //////

+[WX CHANNEL]----+

|CH  |CH   |LOCK-|

| No.| INH |  OUT|

+----+-----+-----+

|  1 |     |     |

|  2 |     |     |

|  3 |     |     |

|  4 |     |     |

|  5 |     |     |

|  6 |     |     |

|  7 |     |     |

|  8 |     |     |

|  9 |     |     |

| 10 |     |     |

+----+-----+-----+

When weather channels are selected with the transceiver, the channels in

the screen table can be used.  When "I" (Inhibit) is selected in the "CH INH"

column, the selected channels do not appear in the selected channel group.

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A SUB menu appears under the TOP menu.

3) Select "WX   ch" with the [W] key, or the [Arrow] and [Enter] keys.

- The WX CHANNEL screen appears.

CH INH

The selected channels do not appear during channel selection.

1) Select the desired channel in the CH INH column with an [Arrow] key.

2) Push the [Space] key to toggle the "I" or no indication selection.

- "I" means inhibit.

- No indication means the channel appears and can be used during

operation.

LOCKOUT

The selected channels are locked out during scan. This setting can also be

performed via the transceiver's front panel.

1) Select the desired channel in the LOCKOUT column with an [Arrow] key.

2) Push the [Space] key to select lockout or non-lockout.

- "L" appears when lockout is specified and no indicator appears when

lockout is canceled.

- 16 -

////// EXPERT SCREEN //////

+[EXPERT]---------+------------+------------+

| TITLE           | CONDITION  |  SETMODE   |

+-----------------+------------+------------+

| SCAN FUNCTION   |  ENABLE    | ---------- |

| SCAN MODE       |  NORMAL    |  APPEAR    |

| SCAN TIMER      |  OFF       |  APPEAR    |

| DUAL/TRI MODE   |  TRI-WATCH |  APPEAR    |

| AMPLITUDE 1300Hz|         25 | ---------- |

| AMPLITUDE 2100Hz|         50 | ---------- |

| WATCH TIMER(ms) |        125 | ---------- |

+-----------------+------------+------------+

This screen is available when "/expert" is included in the program startup

command. This screen's contents are separated from the common screen to help

prevent programming errors.

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A SUB menu appears under the TOP menu.

3) Select "EXPERT" with the [E] key, or the [Arrow] and [Enter] keys.

- The EXPERT screen appears.

SCAN FUNCTION

The scan function is inhibited in some transceiver versions. This item is

used to enable or disable transceiver scanning and to add or remove the

scan item from the transceiver's SET mode.

Selectable value:    ENABLE or DISABLE

Selection method:    Push [Space] to toggle the selection.

SCAN MODE

This item sets the scan function to normal scan or priority scan. This

setting can also be performed using the transciever's SET mode. When

selecting "DISAPPEAR" in the SETMODE column, ONLY the selected scan type

can be used.

Selectable value:     NORMAL or PRIORITY (in the CONDITION column)

APPEAR or DISAPPEAR (in the SETMODE column)

Selection method:     Push [Space] to toggle the selection.

- 17 -

SCAN TIMER

This item sets the scan resume condition. When the scan timer is ON, scan

pauses for 5 sec. with receiving a signal.  When the scan timer is OFF,

scan pauses until a signal disappears. This setting can also be performed

using the transceiver's SET mode. When selecting "DISAPPEAR" in the

SETMODE column, this setting cannot be performed using the transceiver's

SET mode.

Selectable value:    ON or OFF (in the CONDITION column)

APPEAR or DISAPPEAR (in the SETMODE column)

Selection method:    Push [Space] to toggle the selection.

DUAL/TRI MODE

This item sets the transceiver's watch function to dual or tri-watch. This

setting can also be performed using the transceiver's SET mode. When select-

ing "DISAPPEAR" in the SETMODE column, ONLY the selected watch function can

be used.

Selectable value:    DUALWATCH or TRI-WATCH (in the CONDITION column)

APPEAR or DISAPPEAR (in the SETMODE column)

Selection method:    Push [Space] to toggle the selection.

AMPLITUDE 1300Hz

DSC transmissions consist of 1300 Hz and 2100 Hz AFSK signals. This item is

used to set the deviation of the 1300 Hz components in the AFSK signal.

Selectable value:    0 to 100

Selection method:    Push the digit keys, then [Enter] to input the

desired value.

**                                                       **

** NOTE: DO NOT change this item from its default value  **

**       of 25. Otherwise, your DSC call cannot be       **

**       decoded by other parties.                       **

**                                                       **

AMPLITUDE 2100Hz

This item is used to set the deviation of the 2100 Hz components in the

ASFK signal.

Selectable value:     0 to 100

Selection method:     Push the digit keys, then [Enter] to input the

desired value.

**                                                       **

** NOTE: DO NOT change this item from its default value  **

**       of 50. Otherwise, your DSC call cannot be       **

**       decoded by other parties.                       **

**                                                       **

- 18 -

WATCH TIMER(ms)

This item sets the watching time for the transceiver's dual/tri-watch

function. The pausing time for this function is fixed at 1.5 sec. This

item also sets scanning times.

Selectable value:    1 to 255

Selection method:    Push the digit keys, then [Enter] to input the

desired value.

**                                                       **

** NOTE: DO NOT change this item from its default value  **

**       of 125 (msec.). Otherwise, scan may not         **

**       function correctly.                             **

**                                                       **

- 19 -

-------------------------------------------------------------------------------

4. CLONING OPERATION

-------------------------------------------------------------------------------

Programmed data can be cloned to an IC-M58 instantly.  All data is clone sent

from PC to transceiver (PC->TR) or clone read from transceiver to PC (PC<-TR)

for approx. 15 sec.  A cloning cable, the OPC-478, is needed to connect a PC's

RS-232C port to the transceiver.

For convenience, when changing a small portion of the programmed contents,

clone read the data from the transceiver, modify the data, then clone send

it back to the transceiver.

CLONE SEND

The programmed data in the PC is copied to the transceiver.

1) Connect the OPC-478 between the PC's RS-232C terminal and transceiver's

[SP] jack.

2) Turn the PC and transceiver power ON.

3) Start up this program.

4) Select "Clone" in the TOP menu with the [C] key, or the [Arrow] and

[Enter] keys.

- Clone sub menu appears.

5) Select "PC->TR" in the SUB menu with the [P] key, or the [Arrow] and

[Enter] keys.

- "Sending oooooo" appears in the bottom left of corner of the screen.

- "CL" and "BUSY" appear on the transceiver display.

- When cloning is finished, "BUSY" disappears.

**                                                        **

**  NOTE: When the wrong port is selected on the RS-232C  **

**        port, "ERROR: CLONE error" appears in the       **

**        bottom left corner of the screen.  In this      **

**        case, check the selection as described below.   **

**        Also, operating the transceiver, such as        **

**        rotating the transceiver's channel selector     **

**        knob just before hitting the [Enter] key may    **

**        result in a cloning error.                      **

**                                                        **

6) Turn the transceiver OFF and ON again.

- Clone send is now complete.

- 20 -

CLONE READ

The pre-set data in the transceiver is copied to the PC.

1) Connect the OPC-478 between the PC's RS-232C terminal and transceiver's

[SP] jack.

2) Turn the PC and transceiver power ON.

3) Start up this program.

4) Select "Clone" in the TOP menu with the [C] key, or the [Arrow] and

[Enter] keys.

- Clone sub menu appears.

5) Select "PC<-TR" in the sub menu with the [T] key, or the [Arrow] and

[Enter] keys.

- "CL" and "TX" appear on the transceiver's display.

- When cloning is finished, the transceiver's display returns to

previous indication.

- When cloning is finished, "BUSY" disappears.

- Turn the transceiver OFF and ON to return to the previous display.

RS-232C

When the connected PC has 2 or more RS-232C ports, the desired port can be

selected with this program.  When the PC has only 1 port, "PORT #1" must be

selected, otherwise, the cloning operation cannot be performed.

1) Select "Clone" in the TOP menu with the [C] key, or an [Arrow] and

[Enter] keys.

- Clone sub menu appears.

2) Select "RS-232C" in the SUB menu with the [R] key, or the  [Arrow] and

[Enter] keys.

- RS-232C port selection block appears.

3) Select the desired port number to connect the cloning cable with the

[Arrow] and [Enter] keys.

- Selected number is the same as the PC port number assignment.

- 21 -

-------------------------------------------------------------------------------

5. FILE SAVING AND LOADING

-------------------------------------------------------------------------------

PROGRAM INFORMATION

The EX-1682 contains the following:

EX1682.EXE    : Main program

M58EUR.INI    : Europe version default settings

M58UK.INI     : UK version default settings

M58USA.INI    : USA version default settings

M58HOL.INI    : Holland version default settings

M58FRA.INI    : France version default settings

M58.DOC       : This document file

When starting up this program and no version name, such as USA, EUR, UK, HOL

or FRA is used, or an invalid version name is used, USA.INI is loaded.

FILE SAVING OPERATION

1) Push [Esc] to indicate the TOP menu.

2) Select "File" in the TOP menu with the [F] key, or the [Arrow] and

[Enter] keys.

- File sub menu appears.

3) Select "Save" in the sub menu with the [S] key, or [Arrow] and

[Enter] keys.

- A file name box appears and shows "FILE: noname.icf".

4) Type the desired file name to be saved.

- ICF is recommended as a file extension for loading convenience.

- When no file extension is used, ".ICF" is automatically added to the

file name.

- The TOP menu and the file name disappear and the previous menu appears.

**                                                                        **

**  SAVING NOTE:                                                          **

**                                                                        **

**  An initial setting file name such as "M58USA.INI" cannot be used      **

**  as a saved file name, since overwriting the initial setting is        **

**  inhibited.                                                            **

**                                                                        **

**  When starting up the program and no file name has been loaded,        **

**  "noname.icf" appears in the file name box.  In this case, direct      **

**  file name entry is possible ("noname" disappears automatically).      **

**  However, once a file is loaded or saved, file name entry does not     **

**  change the file name. ([Del] or [Ins] key must be used to change the  **

**  file name.)                                                           **

**                                                                        **

**  To indicate the previously saved file name, "*" (wild card) may be    **

**  helpful to check the file name.                                       **

**                                                                        **

- 22 -

FILE LOADING OPERATION

1) Push [Esc] to indicate the TOP menu.

2) Select "File" in the TOP menu with the [F] key, or the [Arrow] and

[Enter] keys.

- File sub menu appears.

3) Select "Load" in the SUB menu with the [L] key, or [Arrow] and

[Enter] keys.

- A file name box appears and shows "FILE: *.ICF".

4) Type the desired file name, then push [Enter].

Or, push [Enter] to indicate the pre-programmed file name, select the

desired file name with an [Arrow] key, then push the [Enter] key twice.

- The typed or selected file is loaded, then the file name box

disappears.

- The loaded file name appears in the bottom left corner of the screen.

DOS COMMAND

In the "File" sub menu, a "Dos" item appears.  This command pauses the

program so that a DOS command can be used.  When you want to return to

normal operation of the program, type "EXIT", then push [Enter].

- 23 -

-------------------------------------------------------------------------------

6. PRINTING

-------------------------------------------------------------------------------

The set data can be printed out for convenience and reference for subsequent

programming.  Only the currently displayed screen is printed out, therefore,

the printout operation must be performed for each screen desired.

1) Push [Esc] to indicate the TOP menu.

2) Select "Print" in the TOP menu with the [P] key, or the [Arrow] and

[Enter] keys.

- Print sub menu appears and shows "Current".

3) Push [Enter] again to print out the currently displayed data.

4) After printing is complete, the TOP and SUB menus disappear.

- 24 -