EX-1726  FIELD PROGRAMMING SOFTWARE

For the IC-M710

Copyright 1991-1995 Icom Inc.

Portions of the software copyrighted by Microsoft Corporation 1982-1988

**                                                       **

**  This program is designed to perform cloning for the  **

**  IC-M710 MF/HF TRANSCEIVER only.                      **

**  Before using the program, make a backup copy of the  **

**  original disk.                                       **

**                                                       **

------------------------------------------------------------------------------

TABLE OF CONTENTS

------------------------------------------------------------------------------

1. PROGRAM START UP AND SHUT DOWN ................................. 2

2. OPERATION ...................................................... 3

3. SCREEN DESCRIPTION ............................................. 4

COMMON SCREEN ................................................ 4

USER CH SCREEN .............................................. 12

ITU SIMP SCREEN ............................................. 14

FREQ RANGE SCREEN ........................................... 16

MODE SCREEN ................................................. 18

4. CLONING OPERATION ............................................. 20

5. FILE SAVING AND LOADING ....................................... 22

6. PRINTOUT ...................................................... 24

- 1 -

------------------------------------------------------------------------------

1. PROGRAM START UP AND SHUT DOWN

------------------------------------------------------------------------------

1) Boot up DOS.

2) Insert the EX-1726 backup disk.

3) Type the start command:

**                                        **

**  EX-1726 /(option) /(version) [Enter]  **

**                                        **

1. #01, #02, #03, #11, #12 can be used as "(version)".

#01: U.S.A. General version

#02: U.S.A. Marine version

#03: U.S.A. GMDSS version

#11: Europe Marine version

#12: Europe GMDSS version

2. When the version is specified, the corresponding

default settings are read.

3. When no version is specified, data is read from the M710#01.INI (for

U.S.A. General version) file.

4. When "expert" is used as "(option)," several settings are available

for programming additional items.

5. When a file name is used instead of "/(version)," the specified file

is loaded at start up.

4) The COMMON screen appears.

**                                                              **

**  The loaded file name appears in the bottom left corner of   **

**  the screen.  "NONAME.ICF" appears when no data file is      **

**  loaded.                                                     **

**                                                              **

5) When programming is completed:

push [Esc] (indicates the TOP menu)

then [F] (selects file SUB menu)

then [S] (saves the set data; if not necessary skip this operation.)

then [Q] (quits the program)

- 2 -

------------------------------------------------------------------------------

2. OPERATION

------------------------------------------------------------------------------

The EX-1726 has a TOP menu and 6 screens for programming.  All screens or

operations can be accessed from the TOP menu.  A SUB menu appears under the

TOP menu when an item in the top menu is selected with an [Arrow] key and

[Enter].  The TOP menu appears when the [Esc] key is pushed.

1) After the program starts up, push the [Esc] key to indicate the top menu.

2) Push a highlighted character; or an [Arrow] key and the [Enter] key to

select the desired item.

- A SUB menu appears under the selected item.

3) Push a highlighted character; or an [Arrow] key and the [Enter] key to

select the desired screen.

- The desired screen appears.

4) Select the desired item with an [Arrow] key, then push [Space] or the

digit keys to set the item.

- See descriptions of each screen for details.

- 3 -

------------------------------------------------------------------------------

3. SCREEN DESCRIPTIONS

------------------------------------------------------------------------------

////// COMMON SCREEN //////

The COMMON screen automatically appears after starting up this program.

This screen allows you to set conditions common to all channels.

+[Common]---------+------------++-----------------+------------+

| ITEM            |            || ITEM            |            |

+-----------------+------------++-----------------+------------+

| Tx meter        |  RF        || Indication type | COMMENT    |

| Tx power select | low,mid,hi || Narrow filter   | OFF        |

| Mode sel on 2182|  ENABLE    || FSK shift [Hz]  | 170        |

| Max user ch No. |       160  || FSK polarity    | NORMAL     |

| ITU FSK ch      |  USABLE    || CW break-in     | FULL       |

| ITU direction   |  NORMAL    || ACC mod. input  | ACC SEND   |

| Antenna tuner   |  AT-130/E  || Mic audio input | PTT        |

| Auto tuning use |  OFF       || LCD contrast    |         7  |

| Auto tuning type|  PTT       || NMEA ID         |        01  |

| ALM timer [sec] |        50  || NMEA input jack | DSC/REMO   |

| 2182 sel at ALM |  AUTOMATIC |+-----------------+------------+

| Scan type       |  CHANNEL   || Cross ch oper.  | DISABLE    |

| Scan ref.[msec] |       130  || ITU ch use      | ENABLE     |

| Scan speed[step]|         4  || Freq. program   | RX ONLY    |

+ ----------------+------------++-----------------+------------+

SCREEN SELECTION

1) Push the [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or an [Arrow] and [Enter] keys.

- A sub menu appears under the TOP menu.

3) Select "Common" with the [C] key, or an [Arrow] and [Enter] keys.

- The COMMON screen appears.

- 4 -

TX METER (Tx meter)

The bar-indicator on the display can be used as an RF meter or antenna

current meter.  European regulations require an antenna current meter.

For the antenna current meter to function properly, the AT-130E HF

AUTOMATIC ANTENNA TUNER must be connected to the IC-M710.

Selectable value:    RF ...... RF meter

ANT C ... Antenna current meter

Selection method:    Push [Space] key to toggle the selection.

TX POWER SELECTION (Tx power select)

The IC-M710 has 3 output powers available.  However, some countries do not

allow low power.  In such cases output power should be restricted using

this item.

Selectable value:    low,mid,hi .... 20, 60, 150 W

mid,high ...... 60, 150 W

high .......... 150 W only

Selection method:    Push [Space] or [Backspace] for selection.

MODE SELECTION ON THE EMERGENCY CHANNEL (Mode sel on 2182)

When pushing the [2182KHz] switch, the 2182 kHz emergency frequency can be

selected instantly as channel 0.  When selecting the emergency channel,

the operating mode selection can be inhibited, if desired.

Selectable value:    ENABLE or DISABLE

Selection method:    Push [Space] to toggle the selection.

MAXIMUM USER CHANNEL NUMBER (Max user ch No.)

The maximum number of user channels for the IC-M710 is 160.  However,

this number can be reduced for easier operation.

Selectable value:    1 to 160

Selection method:    Enter the desired number, then push [Enter],

or push [Space] to select 160.

ITU FSK CHANNEL (ITU FSK ch)

ITU FSK channels appear as a group between the ITU 25 MHz band and user

channels.  This FSK channel group can be hidden for voice communication use

only.  This setting can be modified in the transceiver's set mode.

Selectable value:    USABLE or INHIBIT

Selection method:    Push [Space] to toggle the selection.

- 5 -

ITU DIRECTION (ITU direction)

The IC-M710 is designed for ship-to-shore operation when ITU channels are

in use.  When using the transceiver as a coastal station, the duplex

direction must be reversed for shore-to-ship operation.

Selectable value:    NORMAL ..... For vessel use

REVERSE .... For coastal use

Selection method:    Push [Space] to toggle the selection.

AUTOMATIC TUNING CONDITION (Auto tuning use)

When the optional AT-130 or AT-130E AUTOMATIC ANTENNA TUNER is connected,

tuning can be started automatically without the [TUNE] switch for instant

operation.  This setting can be modified in the transceiver's set mode.

Selectable value:    ON or OFF

Selection method:    Push [Space] or [Backspace] for selection.

TUNING START METHOD (Auto tuning type)

When the automatic tuning condition is set to ON, the start trigger is

selected in this item.  "PTT" tunes the antenna when pushing the PTT on the

new frequency.  "FREQUENCY" tunes the antenna 2.5 sec. after a new

frequency is selected.

Selectable value:    PTT, FREQUENCY

Selection method:    Push [Space] to toggle the selection.

ALARM TIMER (ALM timer [sec])

During on distress a 2-tone emergency signal should be transmitted for at

least 30 sec. before the emergency call.  The transceiver has a timer which

automatically stops the 2-tone alarm after a specified time.  This item

sets the timer period.

Selectable value:    1 to 99 (sec.)

Selection method:    Enter the desired period, then push [Enter]

or push [Space] to select "50".

AUTOMATIC 2182 kHz SELECTION AT ALARM TRANSMISSION (2182 sel at ALM)

To transmit the 2-tone alarm signal, 2182 kHz must be selected in advance.

However, automatic 2182 kHz selection is available when pushing and

holding [ALARM] and [TX FREQ].

Selectable value:    MANUAL ...... 2182 kHz must be selected manually

AUTOMATIC ... 2182 kHz is selected automatically

Selection method:    Push [Space] to toggle the selection.

** NOTE: When selecting "AUTOMATIC" in this item, testing alarm **

**       transmission on a different frequency is not possible. **

- 6 -

SCAN TYPE (Scan type)

This item selects one of the scan functions.

Channel scan and channel resume scan search the 5 channels around the

selected channel when a user channel is selected or search all ITU channels

in the band when an ITU channel is selected.

Channel scan is canceled when transmitting and channel resume scan pauses

for 30 sec. when transmitting.

Programmed scan searches for signals over the frequency range at high

speed while squelch is closed and at low speed while squelch is open. The

frequency range of programmed scan is between user channels maximum number

and maximum number minus 1 (e.g. 159 and 160).

Selectable value:    CHANNEL, CH RESUME, PROGRAM, or OFF(scan inhibit)

Selection method:    Push [Space] or [Backspace] for selection.

**                                                               **

** NOTE: When the "Freq. program" item is selected as "INHIBIT", **

**       "PROGRAM" does not appear as selectable values.         **

**                                                               **

SCAN SPEED REFERENCE [msec./ch] (Scan ref.[msec])

Scan speed can be selected in the next item and in the transceiver's set

mode.  The reference value of the selection can be set in this item.  The

reference value is the time paused on a channel and is used for the fastest

scan speed.  This selection AFFECTS CHANNEL SCAN ONLY.

Selectable value:    100 to 200 in 10 msec increments.

(100 is the fastest speed)

Selection method:    Enter the desired period, then push [Enter]

or push [space] to select "130".

- 7 -

SCAN SPEED (Scan speed[step])

Scan speed is selectable from one of 10 steps.  Scan speed "1" is the

fastest speed.  This setting can be modified in the transceiver's set mode.

+----------------------+--------+-------+-------+-------+--------+---------+

|Selection [step]      |    1   |   2   |   3   |  4    | 5....9 |   10    |

+----------------------+--------+-------+-------+-------+--------+---------+

|Channel scan [msec/ch]| 1 x ref|2 x ref|4 X ref|8 x ref| ...... |512 x ref|

+----------------------+--------+-------+-------+-------+--------+---------+

(ref stands for the value of "scan speed reference"

on the above item.)

+----------------------------+---+---+---+---+---+---+---+---+---+---+

|Selection [step]            | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10|

+----------------------------+---+---+---+---+---+---+---+---+---+---+

|Channel resume scan [sec/ch]| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10|

+----------------------------+---+---+---+---+---+---+---+---+---+---+

|Programmed scan* [msec/ch]  | 10| 20| 40| 60| 90|140|210|320|480|720|

+----------------------------+---+---+---+---+---+---+---+---+---+---+

*value shows while squelch open; fixed 10 msec/ch while squelch closed.

Selectable value:    1 to 10

Selection method:    Enter the desired number, then push [Enter].

or push [Space] to select "4".

INDICATION TYPE (Indication type)

The display indication type can be toggled with the [CH/FREQ] key.  This

item sets which display types are toggled.  This setting can be modified in

the transceiver's set mode (except "FREQ ONLY" setting).

Selectable value:

COMMENT ...... Channel number/comment and Rx/Tx frequencies

FREQ ONLY .... Channel number/frequency and Rx/Tx frequencies

(Comment cannot be selected on the transceiver's

set mode.)

FREQUENCY .... Channel number/frequency and Rx/Tx frequencies

(Comment can be selected on the transceiver's set

mode.)

Selection method:    Push [Space] or [Backspace] for selection.

- 8 -

NARROW FILTER (Narrow filter)

This item selects the passband width when selecting A1A(CW), FSK or J2B

mode.  These narrow widths are available for the GMDSS versions and an

optional filter is required for the Marine/General versions.

This setting can be modified in the transceiver's set mode.

Selectable value:    OFF ..... 2.3 kHz/-6 dB

ON ...... 350 Hz/-6 dB (GMDSS versions)

500 Hz/-6 dB (optional for

Marine/General versions)

Selection method:    Push the [Space] key to toggle the selection.

**                                                       **

** NOTE: When "ON" is set without an optional filter     **

**       installation for the General/Marine versions,   **

**       the transceiver's transmit/receive circuitry do **

**       not function.                                   **

**                                                       **

FSK SHIFT FREQUENCY (FSK shift [Hz])

For FSK operation, two audio frequencies, mark and space, are used to send

data.  The shift frequency is the difference between these frequencies.

This setting can be modified in the transceiver's set mode.

Selectable value:    170, 425, 850

Selection method:    Push [Space] or [Backspace] for selection.

or, enter a number above via the keyboard, then

push [Enter].

FSK POLARITY (FSK polarity)

Normal and reverse polarity are available for FSK operation.  This setting

can be modified in the transceiver's set mode.

Selectable value:    NORMAL ...... key open: mark,   key close: space

REVERSE ..... key open: space,  key close: mark

Selection method:    Push the [Space] key to toggle the selection.

CW BREAK-IN (CW break-in)

The CW break-in function (in A1A mode) toggles transmit and receive with CW

keying.  Full break-in allows you to receive signals between transmitted

keying pulses during CW transmission.  Semi break-in allows you to mute

receiving until keying stops with some delay time.  This setting can be

modified in the transceiver's set mode.

Selectable value:    FULL, SEMI, OFF

Selection method:    Push [Space] or [Backspace] for selection.

- 9 -

ACC MODULATION INPUT ACCEPT (ACC mod. input)

Modulation input from the ACC(1) socket, pin 4, is usually acceptable only

when the "SEND" line (ACC(1) pin 3) is grounded.  However, this item can be

used to allow modulation input when the PTT switch is pushed.

Selectable value:    ACC SEND ...... at ACC SEND only

PTT/ACC ....... at both ACC SEND and PTT

Selection method:     Push the [Space] key to toggle the selection.

MICROPHONE AUDIO INPUT ACCEPT (Mic audio input)

Microphone audio is usually acceptable only when the PTT switch is pushed

and held.  However, this item can be used to accept audio when the "SEND"

line (ACC(1) pin 3) is grounded.

Selectable value:    PTT .......... at PTT switch only

PTT/ACC ...... at Both ACC SEND and PTT

Selection method:    Push the [Space] key to toggle the selection.

LCD CONTRAST (LCD contrast)

The LCD contrast can be adjusted through 10 levels to suit transceiver

mounting angle, location and ambient lighting.  This setting can be

modified in the transceiver's set mode.

Selectable value:     1 to 10  (1 is lowest and 10 is highest)

Selection method:     Enter the desired number, then push [Enter]

or push [Space] to select "7".

ID NUMBER FOR REMOTE CONTROL (NMEA ID)

When connecting an external controller such as a personal computer, 2-digit

ID codes are required to access the transceiver.  The IC-M710 adopts

NMEA0183 format and uses a "proprietary sentence" for remote control.  This

setting can be modified in the transceiver's set mode.

Selectable value:    01 to 99

Selection method:    Push digit keys, then [Enter] for ID code entry

or push [Space] to select "01".

**                                                         **

** NOTE: A different NMEA ID code must be used for remote  **

**       control.  For example, avoid "90" which is the    **

**       default setting of the GM-110DSC DSC TERMINAL     **

**       UNIT's ID code.                                   **

**                                                         **

- 10 -

NMEA CONTROL DATA INPUT JACK (NMEA input jack)

Remote control signals can usually be input via the [DSC] (or REMOTE)

socket only.  However, this item can be used to also set the [CLONE] jack

as acceptable in parallel with the [DSC] (or REMOTE) socket.  This setting

can be modified in the transceiver's set mode.

Selectable value:    DSC/REMO,  CLONE

Selection method:    Push [Space] to toggle the selection.

CROSS CHANNEL OPERATION (Cross ch oper.)

("/EXPERT" is necessary in the start up command to edit this item)

Cross channel operation is available for European countries to operate

different channels for receiving and transmitting.  The U.S.A. versions

must be set to "DISABLE" in this setting.

Selectable value:    DISABLE,   ENABLE

Selection method:    Push [Space] to toggle the selection.

ITU CHANNEL USE (ITU ch use)

("/EXPERT" is necessary in the start up command to edit this item)

The IC-M710 has all ITU channels.  However, customers of some versions

such as the General version, may not need the ITU channels.  In such cases,

the ITU channels can be hidden for easier operation.

Selectable value:    DISABLE,   ENABLE

Selection method:    Push [Space] to toggle the selection.

FREQUENCY PROGRAMMING CAPABILITY (Freq. program)

("/EXPERT" is necessary in the start up command to edit this item)

Both Rx and Tx frequencies can be programmed via the IC-M710's keypad.

However, this capability can be restricted to suit regulations, operation

methods, etc.

Selectable value:    RX/TX,  RX ONLY,  INHIBIT

Selection method:    Push [Space] or [Backspace] for selection.

**                                                               **

**  NOTE: If "Scan type" is selected as "PROGRAM" and this item  **

**        sets to "INHIBIT", the scan type becomes "CHANNEL"     **

**        automatically.                                         **

**                                                               **

- 11 -

////// USER CHANNEL SCREEN //////

The USER CH screen allows you to set all user channel and [2182KHz] emergency

channel contents.  These contents can be overwritten from the transceiver's

front panel (except ch 0) when the "Freq. program" item in the COMMON screen

is set as "RX/TX" (or "RX ONLY).

+[USER ch]-------+-----------+------+---------+

|CH  | Rx FREQ.  | Tx FREQ.  | MODE | COMMENT |

| No.|     [kHz] |     [kHz] |      |         |

+----+-----------+-----------+------+---------+

|  0 |   2182.0  |   2182.0  | H3E  | EMERGEN |

|  1 |   5000.0  |           | H3E  | WWV     |

|  2 |  10000.0  |           | H3E  | WWV     |

|  3 |  15000.0  |           | H3E  | WWV     |

|  4 |   2182.0  |   2182.0  | H3E  | EMERGEN |

|  5 |   4125.0  |   4125.0  | J3E  | DISTRES |

|  6 |   6215.0  |   6125.0  | J3E  | DISTRES |

|  7 |   8291.0  |   8291.0  | J3E  | DISTRES |

|  8 |  12290.0  |  12290.0  | J3E  | DISTRES |

|  9 |  16420.0  |  16420.0  | J3E  | DISTRES |

| 10 |   2450.0  |   2003.0  | J3E  | KMI     |

| 11 |   2506.0  |   2406.0  | J3E  | KMI     |

| 12 |   4357.0  |   4065.0  | J3E  | KMI     |

| 13 |   4402.0  |   4110.0  | J3E  | KMI     |

~    ~           ~           ~      ~         ~

~    ~           ~           ~      ~         ~

|160 |           |           |                |

+----+-----------+-----------+----------------+

SCREEN SELECTION

1) Push the [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or an [Arrow] and [Enter] keys.

- A sub menu appears under the TOP menu.

3) Select "User" with the [U] key, or an [Arrow] and [Enter] keys.

- The USER CH screen appears.

- 12 -

RX FREQ./TX FREQ.

The programmed frequencies are used when user channels are selected.

1) Select the desired channel in the "Rx FREQ." or "Tx FREQ." column with a

[PageUp], [PageDown] or [Arrow] key.

2) Enter the desired frequency in kHz (incl. 1 decimal place),

then push [Enter]; or,

push [Space] then [Enter] to set the channel as a blank channel.

-  The acceptable frequency range is determined by the setting in the FREQ

RANGE screen.

-  Blank setting on the Rx freq. erases mode and comment simultaneously.

Blank setting on the Tx freq. produces the transmit inhibit channel.

**                                                                  **

**  NOTE: If "?" appears at the end of the frequency, (1) the       **

**        acceptable frequency range has been changed (decreased)   **

**        in the FREQ RANGE  screen; or (2) the usable mode is      **

**        restricted in the MODE screen.  Check both screens,       **

**        otherwise cloning will not be successful.                 **

**                                                                  **

MODE

Programmed modes are used when user channels are selected.  This column shows

the same contents as the display indication, such as J3E or USB.  The usable

mode and mode indication are set in the MODE screen.

1) Select the desired channel in the "MODE" column with a [PageUp], [PageDown]

or [Arrow] key.

2) Push [Space] or [Backspace] to select the desired operating mode.

Or, enter the desired mode via the keyboard directly.

** The usable modes are determined by the setting in the MODE screen.

COMMENT

The IC-M710 shows channel names with up to 7 characters when channel

indication is selected, to conveniently display station names, operating

purposes, etc.  Check that the "Indication type" in the COMMON screen is set

to "COMMENT" to indicate memory names.

1) Select the desired channel in the "NAME" column with a [PageUp], [PageDown]

or [Arrow] key.

2) Enter the desired channel name via the keyboard.

- Acceptable characters: 0-9, A-Z, ' ( ) * + - < = > @

- Capital letters only although small letters can be input.

- To delete the name, push [Space].

- 13 -

//////  ITU SIMP CH SCREEN //////

The ITU SIMP CH screen allows you to set all ITU simplex channel contents.

These contents can be overwritten from the transceiver's front panel when the

"Freq. program" item in the COMMON screen is set to "RX/TX".

+[ITUsimp ch]------+------+---------+

| CH   |  FREQ.    | MODE | COMMENT |

|  No. |     [kHz] |      |         |

+------+-----------+------+---------+

|  4-1 |   4125.0  | J3E  | 4A LTD  |

|  4-2 |   4146.0  | J3E  | 4B LTD  |

|  4-3 |   4149.0  | J3E  | 4C LTD  |

|  4-4 |   4366.0  | J3E  | ALASKA  |

|  4-5 |   4369.0  | J3E  | ALASKA  |

|  4-6 |   4396.0  | J3E  | ALASKA  |

|  4-7 |   4402.0  | J3E  | ALASKA  |

|  4-8 |   4420.0  | J3E  | ALASKA  |

|  4-9 |   4423.0  | J3E  | ALASKA  |

+------+-----------+------+---------+

|  6-1 |   6224.0  | J3E  | 6A LTD  |

|  6-2 |   6227.0  | J3E  | 6B LTD  |

~      ~           ~      ~         ~

~      ~           ~      ~         ~

| 25-8 |           |      |         |

| 25-9 |           |      |         |

+------+-----------+------+---------+

SCREEN SELECTION

1) Push the [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or an [Arrow] and [Enter] keys.

- A sub menu appears under the TOP menu.

3) Select "ITU simp" with the [I] key, or an [Arrow] and [Enter] keys.

- The ITU SIMP CH screen appears.

- 14 -

FREQ. [kHz]

The programmed frequencies are used when an ITU simplex channel is selected.

1) Select the desired channel in the "FREQ." column with a [PageUp],

[PageDown] or [Arrow] key.

2) Enter the desired frequency in kHz (incl. 1 decimal place),

then push [Enter]; or,

push [Space] then [Enter] to set the channel as a blank channel.

-  The acceptable frequency range is determined by the setting in the FREQ

RANGE screen.

**                                                                  **

**  NOTE: If "?" appears at the end of the frequency, (1) the       **

**        acceptable frequency range has been changed (decreased)   **

**        in the FREQ RANGE screen; or (2) the usable mode is       **

**        restricted in the MODE screen.  Check both screens,       **

**        otherwise cloning will not be successful.                 **

**                                                                  **

MODE

The programmed mode appears when ITU simplex channels are selected.  This

column shows the same contents as the display indication, such as J3E or USB.

The usable mode and mode indication are set in the MODE screen.

1) Select the desired channel in the "MODE" column with a [PageUp], [PageDown]

or [Arrow] key.

2) Push [Space] or [Backspace] to select the desired operating mode.

Or, enter the desired mode via the keyboard directly.

** The usable modes are determined by the setting in the MODE screen.

COMMENT

The IC-M710 shows channel names with up to 7 characters when channel

indication is selected, to conveniently display station names, operating

purposes, etc.  Check that the "Indication type" in the COMMON screen is set

to "COMMENT" to indicate memory names.

1) Select the desired channel in the "NAME" column with a [PageUp], [PageDown]

or [Arrow] key.

2) Enter the desired channel name via the keyboard.

- Acceptable characters: 0-9, A-Z, ' ( ) * + - < = > @

- Capital letters only although small letters can be input.

- To delete the name, push [Space].

- 15 -

////// FREQ RANGE SCREEN //////

The FREQ RANGE screen allows you to change (restrict) the frequency coverage.

The receiver is a general coverage receiver, therefore only one range is

available.  The transmitter frequency range is separated by 11 to fit the

allowed range.  In addition, the frequency ranges for the ITU simplex channels

are also available for operating convenience.

**                                                                  **

** NOTE: The FREQ RANGE screen can be selected without any special  **

**       operation.  However, setting is possible only when the     **

**       "expert" command is used during start up.                  **

**                                                                  **

+[Freq. range]--------+-----------++---------+-----------+-----------+

|  ITEM   | lower     | higher    ||  ITEM   | lower     | higher    |

|         |  edge[kHz]|  edge[kHz]||         |  edge[kHz]|  edge[kHz]|

+---------+-----------+-----------++---------+-----------+-----------+

|   Rx    |    30.000 | 29999.900 || Tx  1st |  1600.000 |  1999.999 |

+---------+-----------+-----------+| Tx  2nd |  2000.000 |  2999.999 |

+---------+-----------+-----------+| Tx  3rd |  4000.000 |  4999.999 |

| ITU  4M |  4000.000 |  4999.999 || Tx  4th |  6000.000 |  6999.999 |

| ITU  6M |  6000.000 |  6999.999 || Tx  5th |  8000.000 |  8999.999 |

| ITU  8M |  8000.000 |  8999.999 || Tx  6th | 12000.000 | 13999.999 |

| ITU 12M | 12000.000 | 13999.999 || Tx  7th | 16000.000 | 17999.999 |

| ITU 16M | 16000.000 | 17999.999 || Tx  8th | 18000.000 | 19999.999 |

| ITU 18M | 18000.000 | 19999.999 || Tx  9th | 22000.000 | 22999.999 |

| ITU 22M | 22000.000 | 22999.999 || Tx 10th | 25000.000 | 27500.000 |

| ITU 25M | 25000.000 | 27500.000 || Tx 11th |           |           |

+---------+-----------+-----------++---------+-----------+-----------+

SCREEN SELECTION

1) Push the [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or an [Arrow] and [Enter] keys.

- The FREQ RANGE screen appears.

- Be sure the "expert" command is used at start up if you want to change

the contents.

- 16 -

RX

This line is used for setting the receiver frequency coverage continuously.

Enter the desired lower and higher edges.

Selectable range:    30.000 to 29999.900 [kHz]

Selection method:    Enter the desired frequency directly via the

numeral keys.

ITU 4M -- ITU 25M

These lines are used to set the receive/transmit range for ITU simplex

channels.  Enter the desired range for each band.

Selectable range:    1600.000 to 29999.900 [kHz]

Selection method:    Enter the desired frequency directly via the

numeral keys.

Tx 1st -- Tx 11th

These lines are used to set the transmit range for user channels.  Input the

desired frequency range from the upper lines as there are 11 lines for band

separation.

Selectable range:    1600.000 to 29999.900 [kHz]

Selection method:    Enter the desired frequency directly via the

numeral keys;

or push [Space] then [Enter] to delete the

frequency range.

**                                                            **

** NOTE: When a frequency range in an upper line is deleted,  **

**       the frequency range lines underneath are erased,     **

**       automatically.                                       **

**                                                            **

- 17 -

////// MODE SCREEN //////

The IC-M710 has the following mode capabilities.  However, some modes can be

restricted to suit operating needs.

+[Mode]--------+---------+---------+

| MODE  |INDI- |   Rx    |   Tx    |

|       |CATION|         |         |

+-------|------+---------+---------+

|  J3E  |  J3E | ENABLE  | ENABLE  |

|  R3E  |  R3E | ENABLE  | ENABLE  |

|  H3E  |  H3E | ENABLE  | ENABLE  |

|  LSB  |  LSB | DISABLE | DISABLE |

|  J2B  |  J2B | DISABLE | DISABLE |

|  FSK  |  FSK | ENABLE  | ENABLE  |

|  A1A  |  A1A | DISABLE | DISABLE |

+-------+------+---------+---------+

SCREEN SELECTION

1) Push the [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or an [Arrow] and [Enter] keys.

- A SUB menu appears under the TOP menu.

3) Select "MODE" with the [M] key, or an [Arrow] and [Enter] keys.

- The MODE screen appears.

- 18 -

INDICATION

The programmed indication appears on the transceiver's display when the

mode is selected.  The following indications are available for [Space]

selection, however, other indications up to 3 characters can be input via

the keyboard.

Selectable value:    J3E ...... J3E,  USB  or other

R3E ...... R3E        or other

H3E ...... H3E,  AM   or other

LSB ...... LSB        or other

J2B ...... J2B,  AFS  or other

FSK ...... FSK,  DFS  or other

A1A ...... A1A,  CW   or other

Selection method:    Push [Space] to toggle the selection,

or enter the desired name directly from the

keyboard.

RX / TX

These columns select usable modes for operation.  When "DISABLE" is

selected in the Rx column, the mode cannot be selected via the

transceiver's front panel.  When "DISABLE" is selected in the Tx column,

transmission is inhibited when the mode is selected even if a transmit

frequency is programmed.

Selectable value:   ENABLE,  DISABLE

(ENABLE cannot be selected in the Tx column when

the Rx column is set as DISABLE.)

Selection method:   Push [Space] to toggle the selection.

**                                                          **

** NOTE: J3E cannot be set to "DISABLE" in the both RX and  **

**       TX columns.                                        **

**                                                          **

- 19 -

------------------------------------------------------------------------------

4. CLONING OPERATION

------------------------------------------------------------------------------

Programmed data can be cloned to an IC-M710 instantly.  All data is clone

sent (PC -> TR) or clone read (PC <- TR).  A cloning cable, OPC-478 is

necessary to connect a PC's RS-232C port and the transceiver.

For convenience, when changing a small portion of the programmed contents,

clone read the data from the transceiver, modify the data, then clone send

it back to the transceiver.

CLONE SEND (PC->TR)

The programmed data in the PC is copied to the transceiver.

1) Connect the OPC-478 between the PC's RS-232C terminal and transceiver's

[CLONE] jack.

2) Turn the PC and transceiver power ON.

3) Start up this program.

4) Select "Clone" in the TOP menu with the [C] key, or an [Arrow] and

[Enter] keys.

- Clone sub menu appears.

5) Select "PC->TR" in the SUB menu with the [P] key, or [Arrow] and

[Enter] keys.

- "Sending oooooo" appears in the bottom left of corner of the screen.

- "CLONE" appears and counting starts from E000 to EFF0 on the

transceiver.

- When cloning is finished and all data send correctly, "Good" appears on

the transceiver display.

**                                                          **

**  NOTE: - When "CLONE error" appears, check the "RS-232C  **

**          port" setting.                                  **

**        - When "Illegal data in" and screen name appears  **

**          "?" may be appears on the illegal data in the   **

**          screen.  Re-program them, or check the freq.    **

**          range screen.                                   **

**                                                          **

6) Turn the transceiver OFF and ON again.

- Clone send is now complete.

- 20 -

CLONE READ (PC<-TR)

The pre-set data in the transceiver is copied to the PC.

1) Connect the OPC-478 between the PC's RS-232C terminal and transceiver's

[CLONE] jack.

2) Turn the PC and transceiver power ON.

3) Start up this program.

4) Select "Clone" in the TOP menu with the [C] key, or [Arrow] and

[Enter] keys.

- Clone sub menu appears.

5) Select "PC<-TR" in the sub menu with the [T] key, or [Arrow] and

[Enter] keys.

- "CLONE" appears and counting starts from E000 to EFF0 on the

transceiver.

- When cloning is finished, the transceiver's display returns to

the previous indication.

- Clone read is now complete.

RS-232C

When the connected PC has 2 or more RS-232C ports, the desired port can be

selected with this program.  When the PC has only 1 port, "PORT #1" must be

selected, otherwise, the cloning operation cannot be performed.

1) Select "Clone" in the TOP menu with the [C] key, or an [Arrow] and

[Enter] keys.

- Clone sub menu appears.

2) Select "RS-232C" in the SUB menu with the [R] key, or an [Arrow] and

[Enter] keys.

- RS-232C port selection block appears.

3) Select the desired port number to connect the cloning cable with an

[Arrow] and [Enter] keys.

- Selected number is the same as the PC port number assignment.

- 21 -

------------------------------------------------------------------------------

5. FILE SAVING and LOADING

------------------------------------------------------------------------------

FILE SAVING OPERATION

1) Push the [Esc] to indicate the TOP menu.

2) Select "File" in the TOP menu with the [F] key, or an [Arrow] and

[Enter] keys.

- File sub menu appears.

3) Select "Save" in the sub menu with the [S] key, or [Arrow] and

[Enter] keys.

- A file name box appears and shows "FILE: noname.icf".

4) Type the desired file name to be saved.

- ICF is recommended as a file extension for loading convenience.

- When no file extension is used, ".ICF" is automatically added to the

file name.

- The TOP menu and the file name disappear and the previous menu appears.

**                                                                        **

**  SAVING NOTE:                                                          **

**                                                                        **

**  When starting up the program and no file name has been loaded,        **

**  "noname.icf" appears in the file name box.  In this case, direct      **

**  file name entry is possible ("noname" disappears automatically).      **

**  However, once a file is loaded or saved, file name entry does not     **

**  change the file name. ([Del] or [Ins] key must be used to change the  **

**  file name.)                                                           **

**                                                                        **

**  To indicate the previously saved file name, "*" (wild card) may be    **

**  helpful to check the file name.                                       **

**                                                                        **

- 22 -

FILE LOADING OPERATION

1) Push the [Esc] to indicate the TOP menu.

2) Select "File" in the TOP menu with the [F] key, or an [Arrow] and

[Enter] keys.

- File sub menu appears.

3) Select "Load" in the SUB menu with the [L] key, or [Arrow] and

[Enter] keys.

- A file name box appears and shows "FILE: FILE: *.ICF".

4) Type the desired file name, then push [Enter].

Or, push [Enter] to indicate the pre-programmed file name, select the

desired file name with an [Arrow] key, then push the [Enter] key twice.

- The typed or selected file is loaded, then the file name box

disappears.

- The loaded file name appears in the bottom left corner of the screen.

DOS COMMAND

In the "File" sub menu, a "Dos" item appears.  This command pauses the

program so that a DOS command can be used.  When you want to return to

normal operation of the program, type "EXIT", then push [Enter]

- 23 -

------------------------------------------------------------------------------

6. PRINTOUT

------------------------------------------------------------------------------

The set data can be printed out for convenience and reference for subsequent

programming.  Only the currently displayed screen is printed out, therefore,

the printout operation must be performed for each screen desired.

1) Push the [Esc] to indicate the TOP menu.

2) Select "Print" in the TOP menu with the [P] key, or an [Arrow] and

[Enter] keys.

- Print sub menu appears and shows "Current".

3) Push [Enter] again to print out the currently displayed data.

4) After printing is complete, the TOP and SUB menus disappear.

- 24 -