GP I/O Lines

The General Purpose (GP) I/O Lines provide electrical connections between the radio and external facilities, e.g. external PTT, external alarm and internal option boards. Details of the connectivity between the GP Lines, the Accessory Connector and the Internal Option Connectors are included in the Service Manual for the radio.

'A2', 4-channel radio models, have limited GP I/O capability. See Appendix B of the GM350 service/product manual for details.

The I/O Assignment combo box enables correct selection for standard option boards and accessories, or 'Free Format' where the dealer must define each line individually.

FREE FORMAT     -   Dealer Defined I/O TRANSCRYPT  -   Use when Transcrypt scrambler option board fitted SMARTRUNK II    -   Use when Selectone Smartrunk II option board fitted ZR350 - SIMPLEX -   Use to configure the GP I/O for simplex telephone interconnect ZR350, ZR320, ZR340 RICK - TX   -   Use to set up correct signals to act as the TX radio in a uni-directional mobile repeater RICK - RX   -   Use to set up correct signals to act as the RX radio in a uni-directional mobile repeater Other Standard Options may be added from time to time.

The GP lines and their directions are identified in Appendix B of the Service Manual together with the functions that can be allocated to each.

Output Function Descriptions

______________________

Carrier Detect The output is asserted whenever carrier is detected and de-asserted when no carrier is present or the radio is in transmit mode.

PL/DPL Detect The output is asserted whenever the correct decode tone PL or digital PL is detected and de-asserted when any other code is detected or the radio is in transmit mode.

Car Radio Mute The output is active whenever the loudspeaker audio is turned on (defined by squelch setting) and remains active for 4 seconds after the loudspeaker is turned off.

Radio Busy The output is asserted whenever carrier is detected or the radio is in transmit mode.

External Alarm The output is asserted when an individual decode has been received with external alarm enabled.

Internal Option Board Enable The microprocessor can control this line on a per channel basis to enable/disable a non-serial bus type option board.

Transcrypt: Coded/Clear Select The output indicates the required coded/clear status to the Transcrypt module High (5V) = Clear Low (0V) = Coded This line will be toggled when the user presses a button designated for this task.

Transcrypt: Key Select Every time the user presses a button designated as Key Select, this line will be pulsed low for 100ms. This will cause the Transcrypt module to change scramble key. The module provides an audible feedback of the key selected.

Input Function Descriptions

_____________________

Channel Steering When channel steering is in use, 'n' input pins will have been configured in channel steering mode where n is 1, 2, 3, or 4. The software takes the state of these inputs (asserted = 1, de-asserted = 0) and brings them together to form an n-bit value in the order GP6 to GP1 (MSB to LSB). When this value is zero, the channel may be freely selected using the channel select buttons. When this value is non-zero, the channel select buttons are locked and the channel is set to correspond to the chosen value. As the maximum value for n is four, the highest channel selectable by this method will be binary 1111, i.e. channel 15. If the chosen value is greater than the number of channels available in the radio, the highest channel number available will be chosen. When the input value is reset to zero, the radio will return to the last user selected channel.

Hook Hook performs the same function as microphone hook. When rear hook is enabled, the front hook signal is ignored, i.e. there is no need to "ground" the front hook when an external mic is being used, as the hook status is taken from the rear hook connection.

PTT (Voice) When voice mode is selected it is assumed that the external PTT is a mechanical switch that requires software debouncing. When a PTT press is detected, TX audio will be routed from the microphone path.

PTT (Data) When data mode is selected, it is assumed that the external PTT is an electronic switch, e.g. a modem, that requires the fastest possible radio response and no software debouncing. When a PTT press is detected, the microphone path will be muted and the TX audio will be routed from the Flat TX audio input.

PTT (Voice & Data) When data and voice mode is selected, it is assumed that the external PTT is an electronic switch, e.g. a modem, that requires the fastest possible radio response and no software debouncing. When a PTT press is detected, both the microphone path and the Flat TX audio input will be enabled and the two signals will be summed inside the AFIC to form the TX signal.

Mute Audio PA When this input is asserted, the audio PA will be muted. When this input is de-asserted, the audio PA will operate under normal software control and follow squelch as required.

Open RX Audio When this input is asserted, the signal at AFIC RX IN, i.e. discriminator audio, will be routed through to the audio PA independent of squelch and signalling detect. When this signal is de-asserted, the audio PA will be closed. This operation occurs only whilst the radio is in RX mode. Operation in TX mode is not affected.

Open AUX RX Audio When this input is asserted, the signal at AFIC AUX_RX_IN, i.e. internal option board audio, will be routed through to the audio PA independent of squelch and signalling detect. When this input is de-asserted, the audio PA will be closed. This operation occurs only whilst the radio is in RX mode. Operation in TX mode is not affected. This signal can be used by an internal options board to provide alert tones.

Open TX Audio When this input is asserted, the signal will be routed from the AFIC TX IN, i.e. the microphone input, through to the audio PA independent of squelch and signalling detect. This operation occurs only whilst the radio is in RX mode. Operation in TX mode is not affected.  This signal can be used by an external device to provide alert tones heard through the radio's loudspeaker.

Open AUX TX Audio When this input is asserted, the signal will be routed from Pin 5 Flat TX audio through to the audio PA independent of squelch and signalling detect. This operation occurs only whilst the radio is in RX mode. Operation in TX mode is not affected.  This signal can be used by an external device to provide alert tones heard through the radio's loudspeaker.

Internal Option Data Ready When this function is programmed into the radio, it is assumed that a serial option board is fitted. A falling edge on this input indicates that the internal option board wants to send some data to the microprocessor. This input will trigger an interrupt as the line may not stay low for a duration long enough to allow polling to work. This line is also used to acknowledge each byte of data sent to the board.

Internal Option Decode When this line is asserted, an internal option board is indicating that the radio has just been called, therefore the radio will act correspondingly by sounding the call alert, flashing LEDs, etc.

Transcrypt: Kill Radio When this line is asserted, all buttons except on/off will be locked and the radio will be permanently muted.

RX AUDIO

________

This feature enables the Rx Audio output path to be specified to suit the form of audio required, i.e. voice or data.  Select FILTERED for speech output and FLAT for data output, i.e. to a modem.