Reference Oscillator

Adjustment of the reference oscillator is critical for proper radio operation. Improper adjustment will not only result in poor operation but also a misaligned radio that will interfere with other users operating on adjacent channels. For this reason, the reference oscillator should be checked every time the radio is serviced. The frequency counter used for this procedure must have a stability of 0.1 ppm or better.

Should the crystal be damaged, it is not practical to replace the component in the field, due to the extreme amount of processing of temperature compensation data required to be programmed into the radio for correct operation. An alternative repair solution, using a frequency element, rather than a crystal is recommended. See the GM350 service manual for details.