Remove both sides of the housing. It may be necessary to heat the side panel with a soldering iron to achieve this.

Carefully remove all the coil formers (Bobbins). The coil earth is soldered to the top of the housing.

NOTE:	Do not attempt to adjust the variable capacitor (I.F. injection). Ensure that all the slugs on each coil (inside the bobbins) are fully in before removal of the bobbins.

Lay out the bobbins in the order of removal from the block.

Remove the aluminium coil partitions to match those on the opposite side at the top.

These locate the bobbins within the housing.

Grind off excess material to leave a smooth finish.

NB:	Wind the coils in the same direction.

The coils should be finished flush with the top of the bobbin, then secure with Cellotape and coat with nail varnish to prevent any movement.

First Receiver Coil: (Next to the aerial input)

Cut off 30 mm of 0.125 mm Enamel coated wire and solder this to the main coil (which is around 125 mm long).

Insert the start, ie the end with the soldered join, into the bobbin and wind both wires in the same direction.

Wind 1.5 turns for the stub coil, then exit the bobbin (through 35°) Wind the main length to 52 turns, starting diagonally opposite the starting turn.

Second Receiver Coil:

Construct the main and the stub as per the previous coil.

Insert the tap at the start (through the side).

Wind 1.75 turns on the stub. Note: This exits at the same point on the bobbin. Leave the

tails at least 25 mm long.

Wind the main length to 52 turns.

Third Receiver Coil:

This is a 1.5 turn tapped coil, identical to the first receiver coil.

Fourth Receiver Coil: (First band pass coil)

This is a 52 turn coil, like the previous coils (but without a stub coil), however the wire must exit the bobbin through the side and not the top. This is due to the way it is mounted in the housing.

Fifth Receiver Coil: (Second band pass coil)

This is identical to the fourth receiver coil.

Sixth Receiver Coil:

This is a main and stub, tapped coil as per the first, second and third receiver coils. Insert the common tap in the bobbin, wind 2 turns and then exit.