| Radio | Function | Command |
| --- | --- | --- |
| FT - 1000 | Hard reset | Flip off BACKUP switch (Inside the top panel window) |
|  | Memory reset | Press & hold (SUB) & (ENTER) & turn power on |
|  | Soft reset | Press & hold (1,5) & (3,5) & turn power on |
| FT – 990 | Hard reset | Flip off BACKUP switch (Inside the top panel window) |
|  | Memory reset | Press & hold (GEN) & (ENT) & turn power on. |
|  | Soft reset | Press & hold (1,5) & (3,5) & turn power on. ( For checking Display and ROM versions) |
| FT – 890 | Hard reset | Press & hold (HAM/GEN) & (CLAR) & turn power on. |
|  | Soft reset | Press & hold ( A/B) & (A = B) & turn power on. ( For checking Display and ROM versions). |
| FT – 767GX | Hard reset | Press & hold ( OFFSET) & turn power on (140.000 – 148.990 MHz) Press & hold (CLAR) & turn power on (1400 – 145.999 MHz) Press & hold (MCK) & turn power on ( 140.00 – 1487.99 MHz) |
|  | 430/440 toggle | Press & hold (0) & turn power on |
| FT – 757 GX | Hard reset | Press & hold (MARKER) & (LINEAR) & turn power on |
| FT – 747 GX | Hard reset | Slide Backup switch towards tuning dial ( Located on bottom of panel) |
| FT – 2400H | Ham/Extendetd RX | Press & hold (UP) & (DOWN) & turn power on |
|  | Memory reset | Press & hold (D/MR) & (F/w) & turn power on |
|  | Factory Defaults | Press & hold (D/MR) & (REV/SKIP) & turn power on & turn off & Press & hold (D/MR) & turn on. |
| FT - 5100 | Factory Defaults | Press & hold (D/MR) & (REV) & turn power on |
| FT - 5200 | Ham/Extendetd RX | Press & hold (MHz) & ( DVS/HOLD) & turn power on |
|  | Factory Defaults | Press & hold (D/MR) & (REV) & turn power on |

Всем  73!